# Gicar Sniffer Capture Index

All captures taken with the La Marzocco Mini running normally. Sniffer: custom ESP32 firmware on separate hardware, connected in parallel to CN11 (not in-line). Connection: COM4, 115200 baud (sniffer USB), 9600 8N1 inverted (Gicar side). Format: one line per byte, prefixed with B (board→machine) or M (machine→board) and timestamp in ms.

## Capture Conditions (all)

- Machine: La Marzocco Mini, DIP switch 1=OFF (simultaneous boiler heating), DIP switch 2=OFF (240V)

- Coffee boiler setpoint: 94–95°C for most captures (see individual notes)

- Steam boiler: ON unless noted

- Capture tool: inline Python one-liner via PowerShell, COM4, 115200, dsrdtr=False, rtscts=False

## File Descriptions

### 01_idle.txt

Condition: Machine idle, no user action. Duration: ~30s Key content: Baseline R40000023 polling at ~760ms intervals. payload[27] alternates 0x60/0x61 (coffee heater cycling). No W commands. Use for: Reference for idle polling cadence and payload format.

### 02_brew_active.txt

Condition: Active espresso shot in progress. Duration: ~20s Key content: Dense autonomous Z-frames (55 bytes each, ~11 Hz). Almost no R-frame poll responses — machine sends Z-frames instead. First capture confirming Z-frame format. Use for: Z-frame bandwidth analysis; baseline for brew detection.

### 03_steam_active.txt

Condition: Steam wand in active use. Duration: ~30s Key content: R-frames with payload[27]=0x70/0x71 (steam boiler active). Normal poll cadence maintained. Use for: Steam boiler state detection in payload[27].

### 04_temp_change.txt

Condition: User changed coffee boiler setpoint via LM app (94°C → 95°C). Duration: ~30s Key content: W0007 0002 03B6 (0x03B6=950 → 95.0°C). Confirms setpoint register. Use for: Setpoint write command reference.

### 05_preinfusion_change.txt

Condition: User changed pre-infusion duration via LM app. Duration: ~30s Key content: W00E2 0001 14 (0x14=20, duration unit unknown). Normal polling otherwise. Use for: Pre-infusion duration register reference.

### 06_preinfusion_off.txt

Condition: User disabled pre-infusion via LM app. Duration: ~30s Key content: W000B 0001 00 (0x00=off). Normal polling. Use for: Pre-infusion enable/disable register reference.

### 07_backflush_cycle.txt

Condition: Backflush cycle initiated via LM app (with blind filter in basket). Duration: ~130s captured. Key content: exactly 6x W00E1 0001 82 at 1s intervals (t=5.2..10.3s, lever DOWN), then NO further pulses; polling drops to ~one R40000023 per 10s during the cycle (the sparse B-channel 20-100s is the app going deliberately quiet, NOT sniffer loss - byte counts match exactly). Cycle runs t=15..95s: dense Z-frames throughout, z_shot_active held through pump pauses, single falling edge at the true end. Use for: Backflush command sequence; quiet-period polling; cycle-end detection. See the Verified Backflush Protocol section below.

### 08_steam_boiler_off_on.txt

Condition: User toggled steam boiler OFF then ON via LM app. Duration: ~30s Key content: W00E1 0001 01 (steam off), W00E1 0001 81 (steam on). payload[27] transitions 0x70→0x60 (steam off) → 0x70 (steam on). Use for: Steam boiler control register reference.

### 09_standby_time_change.txt

Condition: User changed auto-standby timer (20→30→20 min) via LM app. Duration: ~30s Key content: ZERO W commands despite visible app change. Standby time is stored app/cloud-side only. Use for: Confirming that auto-standby duration is NOT configurable via Gicar.

### 10_schedule_change.txt

Condition: User changed scheduled on-time (05:00→06:00) via LM app. Duration: ~30s Key content: W00E1 0001 81 (steam sync write on app reconnect, NOT a schedule command). Zero schedule-related Gicar writes. Use for: Confirming schedule is NOT configurable via Gicar; understanding steam-sync write.

### 11_standby_entry.txt

Condition: Machine entering full standby via LM app. Duration: ~30s Key content: W0000 0001 00 (standby). payload[27] → 0x00 (full off). Post-standby config writes to 0x0400–0x0406. Use for: Standby entry sequence reference.

### 12_standby_toggle.txt

Condition: Full standby cycle: sleep then wake via LM app. Duration: ~60s Key content: Full standby entry + wake sequence. W0000=0x00 (sleep), W0000=0x01 (wake), R400F (status check), config writes. payload[27] transitions: 0x61 → 0x20 → 0x00 (standby) → 0x20 → 0x61 (awake). Use for: Complete standby/wake command reference. Two distinct standby modes confirmed.

### 13_power_on.txt

Condition: Cold machine power-on from mains off. Sniffer started BEFORE machine powered. Duration: 45s (machine boots at ~26s) Key content: Complete boot sequence captured. X00000000D8 probe (at t=26358ms) → X00000001D9 response (19ms later). Then discovery reads: R4010, R4000 (=0x03 LINEA type), R4003 (serial "Sn2011010551"), R4001. Then config burst, then Phase 3 config writes. Use for: X-handshake confirmation; full startup sequence reference. CRITICAL: only way to see X-probe, which never repeats in normal operation.

### 14_temp_shot.txt

Condition: Empty basket shot (~15s), no puck, no pre-infusion (or default setting). Setpoint: 95°C. Shot pulled immediately after machine at temperature. Duration: ~15s (shot duration) Key content: Dense Z-frames (162 frames). ONE R-frame: temp=87°C (0x0366), brew=1, p[27]=0x7B. Temperature dropped from 93.0°C (Z-frame at t=263ms) to 83.0°C (t=14731ms) — live sensor confirmed. Z-frame temperature formula cross-validated (see CAPTURE_ANALYSIS.md). Use for: Z-frame temperature decoding validation; brew detection; p[27]=0x7B (brew state) reference.

### 15_preinfusion_shot.txt

Condition: Empty basket shot with 3s pre-infusion enabled. Shot pulled ~2s into capture. Setpoint: 95°C. Machine at temperature before shot. Duration: ~20s capture window (~18s shot) Key content: Dense Z-frames (190 frames). Two R-frames (pre-shot, t=813ms + t=1573ms): temp=92°C, brew=0, p[27]=0x61. Z-frame temperature plateau at 92.0°C for first ~5s (pre-infusion phase, solenoid only, no pump flow). Temperature drop began at t=8102ms (~5.9s after shot start), consistent with 3s pre-infusion + ~1.9s thermal lag. Z-frame pair[34,35]=0x10 throughout brew; drops to 0x00 at shot end (t=18165ms, temp=84.0°C). Use for: Pre-infusion thermal signature; shot-end detection via pair[34,35]; Z-frame state decoding.

## Format Reference

Each line in a capture file:

B <timestamp_ms> <hex_byte>    ← board→machine (B-channel)

M <timestamp_ms> <hex_byte>    ← machine→board (M-channel)

Timestamps are relative to capture start (ms). All bytes are single characters of the Gicar ASCII protocol. The full Gicar protocol is documented in CAPTURE_ANALYSIS.md.

# Verified Backflush Protocol (2026-07-03, LineaMini_C6 commit 90f6e00)

Reverse-engineered from capture 07 plus four live failure modes on the real machine; implemented and user-verified working in machine.cpp/gicar.cpp/ui.cpp.

1. With the brew lever DOWN, send exactly 6x W00E1 0001 82 (frame W00E100018258) at 1-second intervals. A single pulse never engages the cycle (the one-shot machine_trigger_clean() that existed before 2026-07-03 never worked).

2. Lift the lever only AFTER the burst completes. Any 0x82 pulse that lands on an already-running cycle aborts it after one pump phase (~5s pump, then dead).

3. Stay near-silent for the duration of the cycle: R-polls slowed to one per 10s, no further pulses. Chattering at the machine at the normal 760ms poll cadence also aborts the cycle at a phase boundary.

4. The machine then runs the full cycle alone: ~80s, 10 pump on/off phases (user-counted on the real machine; ~8s per phase). The Z-stream flags the ENTIRE cycle as one continuous shot - z_shot_active stays 0x10 through the pump pauses, so individual phases are invisible in telemetry. The single z_shot_active falling edge (and end of the dense Z-stream) marks true cycle completion.

5. Restore normal 760ms polling after completion/cancel. Stopping the pulses during the initiation burst cancels cleanly; once running, the cycle completes on its own (the STOP path in firmware just stops pulses, restores polling, and instructs the user to lower the lever).

Failure modes observed while deriving this (all reproduce reliably): one-shot pulse = no cycle; continuous 1Hz pulses through the cycle = aborts after phase 1; 6-pulse burst with lever lifted mid-burst = aborts after phase 1; correct burst+quiet but normal 760ms polling = aborts after phase 1.