# Gicar Protocol — Capture Analysis

Analysis of 10 sniffer captures (2026-06-22). Sniffer: lm_sniffer firmware, 9600 8N1 invert=true, 115200 USB.

## Register Map — Write Commands (confirmed from B-channel)

| Register | Len | Values | Function | Source capture |
| --- | --- | --- | --- | --- |
| 0x0000 | 1 | 0x00=standby, 0x01=wake | Machine power state | 12_standby_toggle |
| 0x0007 | 2 | uint16 big-endian, value/10 = °C | Coffee boiler temp setpoint | 04_temp_change |
| 0x000B | 1 | 0x00=off, (0x01=on implied) | Pre-infusion enable | 06_preinfusion_off |
| 0x00E1 | 1 | 0x01=steam off, 0x81=steam on, 0x82=backflush pulse | Boiler/relay control | 07,08,10 |
| 0x00E2 | 1 | 0x14=20 (unit TBD, likely seconds) | Pre-infusion duration | 05_preinfusion_change |
| 0x00FD | 1 | 0x01=enter, 0x00=exit | Test-mode gate (required before 0x80xx relay writes) | firmware analysis |
| 0x0400 | 1 | 0x00 | Standby config byte 0 (written after standby/wake) | 12_standby_toggle |
| 0x0401 | 1 | 0x00 | Standby config byte 1 | 12_standby_toggle |
| 0x0402 | 1 | 0x00 | Standby config byte 2 | 12_standby_toggle |
| 0x0406 | 1 | 0x00 | Standby config byte 3 | 12_standby_toggle |
| 0x400F | 1 | read-only | Machine status/ack (read after standby/wake) | 12_standby_toggle |
| 0x8002 | 1 | 0x00=off | Pump relay (test-mode only) | firmware analysis |
| 0x8003 | 1 | 0x00=off | EROG solenoid (test-mode only) | firmware analysis |

### Temperature setpoint (0x0007)

W 0007 0002 03B6  →  0x03B6 = 950 → 950/10 = 95.0°C

W 0007 0002 03AC  →  0x03AC = 940 → 940/10 = 94.0°C  (implied from payload observed at 94°C)

### Boiler control (0x00E1)

W 00E1 0001 01   →  steam boiler OFF   (08_steam_boiler_off_on)

W 00E1 0001 81   →  steam boiler ON    (10_schedule_change capture, and machine.cpp)

W 00E1 0001 82   →  backflush pulse    (07_backflush_cycle — sent 5× in first 10s of capture)

Backflush sends 0x82 repeatedly at ~1-second intervals for the full 2-minute cycle. The board also writes 0x81 (steam on) when the app re-syncs with the machine (seen in schedule capture).

### Pre-infusion (0x00E2 + 0x000B)

W 00E2 0001 14   →  0x14=20 → pre-infusion duration (unit unknown; app range 2–20 implies seconds)

W 000B 0001 00   →  0x00  → pre-infusion disabled

When enabled, 0x000B is presumably 0x01. The board periodically reads 4 bytes from 0x00E2 (R00E20004 in startup burst) — likely [duration, unknown, unknown, unknown].

## Poll Protocol

### Board → Machine (B-channel)

R40000023 DB      (11 bytes)   periodic poll, ~760ms interval

Boot/startup sequence (confirmed from 13_power_on capture, 2026-06-22):

Phase 1 — X handshake + discovery (sent once after machine boot, ~26s post power-on):

X00000000D8                   ← X probe: "are you in application mode?"

R40100001D8                   ← read 1 byte from 0x4010 (type check)

R40000001D7                   ← read 1 byte from 0x4000 (machine type = 0x03)

R4003000CEC                   ← read 12 bytes from 0x4003 = serial "Sn2011010551"

R40010001D8                   ← read 1 byte from 0x4001 (= 0xD4)

Phase 2 — Full config read + begin polling:

[multi-read burst: R0000/R0020/R0310/R0300/R00E2] + R40000023DB × 2

Phase 3 — Config sync writes (~5s after polling begins):

W00020001003A                 ← write 0x00 to 0x0002

R00000020D4                   ← re-read identity block

W03100001003C                 ← write 0x00 to 0x0310

W030000072433150116057EBC     ← write 7-byte mode config to 0x0300

Machine responds X00000001D9 exactly 19ms after receiving X00000000D8 (at t=26377ms vs 26358ms). X handshake is one-time per power cycle; not repeated during normal operation (B-X=1, M-X=1 in power-on capture only; all 12 normal-operation captures have B-X=0, M-X=0). Startup burst (board sends on boot and periodically every ~12s):

R0000 0020 D4     read 32 bytes  from 0x0000 (basic config)

R0020 004C EB     read 76 bytes  from 0x0020 (extended config)

R0310 001D EB     read 29 bytes  from 0x0310 (unknown block)

R0300 0007 DC     read  7 bytes  from 0x0300 (unknown block)

R00E2 0004 ED     read  4 bytes  from 0x00E2 (pre-infusion block)

### Machine → Board (M-channel) — R-frame response (81 bytes)

R40000023 [70 hex chars] [CS 2 chars]     ← raw, NO Z prefix

Confirmed from sniffer: zero Z-bytes (0x5A) in M-channel during idle, steam, temp change, pre-infusion, standby-change, and schedule-change captures. The machine sends raw R-frames in response to polls in normal operation. See "Active-Brew Autonomous Stream" below for Z-frame mode.

35-byte payload decoded from hex chars at positions [9..78]:

| Payload byte(s) | Value | Meaning |
| --- | --- | --- |
| [3..14] | "Sn2011010551" (ASCII) | Serial number — sanity check anchor |
| [17] | 0x01=brewing, 0x00=idle | Brew active flag (confirmed from live shot) |
| [27] | see below | Boiler state + heating element |
| [28..29] | uint16 big-endian, /10 = °C | Coffee boiler temperature |

### payload[27] bit map (confirmed from idle/steam/standby/brew captures)

0x60 = 0110 0000   steam boiler OFF, coffee heater OFF

0x61 = 0110 0001   steam boiler OFF, coffee heater ON   (normal idle cycling)

0x70 = 0111 0000   steam boiler ON,  coffee heater OFF

0x71 = 0111 0001   steam boiler ON,  coffee heater ON

0x7B = 0111 1011   BREWING — steam on, heater on, pump active, solenoid open

0x00 = 0000 0000   full standby (both heaters completely off)

0x20 = 0010 0000   transitional / fan only (flickers during standby entry)

- bit 0 (0x01): heating element active

- bit 1 (0x02): pump active (set during brew — confirmed from 14_temp_shot)

- bit 3 (0x08): solenoid open (set during brew — confirmed from 14_temp_shot)

- bit 4 (0x10): steam boiler enabled

- bits [6:5] (0x60): constant base value in normal operation

## Active-Brew / Backflush Autonomous Stream

During active brew AND during backflush, the machine additionally sends 55-byte autonomous Z-frames at ~11 Hz:

Z [seq] [53 bytes of content]     (total 55 bytes)

These appear interleaved with normal poll responses during backflush (07: Z=889, R-frames=62). During brew capture the board barely polled (2 polls), so only autonomous frames were visible.

### Z-frame structure (decoded from 14_temp_shot + 15_preinfusion_shot — 2026-06-22)

Byte   Content

[0]    0x5A 'Z'                           frame marker

[1]    machine ID (0x36='6' on this Mini)

[2..5] 4-char address field "0000"

[6..53] 48 bytes ASCII-hex-encoded content (24 decoded bytes)

[54]   1-byte terminator / checksum

All 48 content bytes are ASCII hex characters. Decoded as 24 contiguous hex pairs (pair n = bytes [6+2n, 7+2n]):

| Pair index | Raw bytes | Decoded (hex) | Meaning |
| --- | --- | --- | --- |
| 0 | [6,7] | 0x01 | constant |
| 1 | [8,9] | 0x65 | constant |
| 2..10 | [10..27] | various | constant — meaning unknown |
| 11 | [28,29] | 0x53 | constant |
| 12 | [30,31] | 0x53 | constant |
| 13 | [32,33] | 0x10 | constant during brew |
| 14 | [34,35] | 0x10 → 0x00 | shot-active flag: 0x10=brewing, 0x00=shot stopped |
| 15 | [36,37] | 0x00 | constant |
| 16 | [38,39] | 0x00 | constant |
| 17 | [40,41] | rolling counter | changes every frame (mod-256 counter stepping ~16/frame) |
| 18..19 | [42..45] | 0x00 | constant |
| 20 | [46,47] | temp high byte | see temperature formula below |
| 21 | [48,49] | temp low byte | see temperature formula below |
| 22 | [50,51] | 0x00 | constant |
| 23 | [52,53] | checksum byte | changes with content |
| — | [54] | terminator | single ASCII char (not a pair) |

### Z-frame temperature formula (confirmed 2026-06-22)

pair_20 = int(chr(frame[46]) + chr(frame[47]), 16)   # high byte

pair_21 = int(chr(frame[48]) + chr(frame[49]), 16)   # low byte — always a multiple of 32

uint16  = (pair_20 << 8) | pair_21

temp_C  = uint16 / 160.0                             # resolution: 0.2°C per step

Confirmed by cross-referencing against R-frame payload[28..29]:

- cap14 brew start: uint16=14880 → 93.0°C (drops to 83.0°C over 14s empty-basket shot)

- cap15 brew start: uint16=14720 → 92.0°C ✓ matches R-frame 0x0398=920/10=92.0°C

- cap14 at t~10725ms: uint16=13920 → 87.0°C ✓ matches R-frame 0x0366=870/10=87.0°C

### Pre-infusion thermal signature (detected indirectly)

During pre-infusion (solenoid open, pump off), temperature holds flat. When pump engages, cold water flow begins and temperature starts dropping.

In cap15 (3s pre-infusion): temperature held at 92.0°C from t=2217ms to t=7199ms (~5s), then dropped to 91.0°C. Pump engaged at ~t=5200ms (t_start + 3s), thermal lag ~1.9s.

This means Z-frame temperature monitoring can detect pump engagement indirectly: temperature stable + pair[34,35]=0x10 = pre-infusion phase (solenoid only) temperature dropping + pair[34,35]=0x10 = extraction phase (pump running)

### Shot lifecycle in Z-frames

| Event | pair[34,35] | temperature |
| --- | --- | --- |
| Brewing (all phases) | 0x10 | varies (see above) |
| Shot stopped | 0x00 | lowest value |
| Idle (no Z-frames sent) | — | — |

Note: the shot-start transition (0x00→0x10) is not observed — Z-frames only appear during active brew, so pair[34,35] is already 0x10 in the first frame of every capture.

## Standby / Wake (confirmed from 12_standby_toggle.txt)

Register 0x0000 controls machine power state:

Standby entry:  W0000 0001 00 38   (data=0x00)

Wake:           W0000 0001 01 39   (data=0x01)

Status check:   R400F 0001 ED      (read 1 byte, sent after both commands)

Post-standby config writes (board sends these ~3s after standby entry and ~1s after wake):

W0400 0001 00 3C

W0401 0001 00 3D

W0402 0001 00 3E

W0406 0001 00 42

These addresses (0x0400–0x0406) appear to be the board syncing standby-related config. All written with 0x00 during standby. Meaning of individual bytes TBD.

payload[27] in standby = 0x00 (both heaters completely off; flickers 0x20 briefly — possibly fan). Normal idle = 0x60/0x61. Wake: board follows with W00E1=0x81 to re-enable steam boiler.

### Two distinct "standby" modes observed

| Mode | Trigger | payload[27] | Description |
| --- | --- | --- | --- |
| Steam-off / energy-save | Auto-timer (no board command) | 0x60 | Steam boiler off, coffee stays warm |
| Full standby | App manual → W0000=0x00 | 0x00 | Both heaters off, machine powered down |

Auto-standby timer fires machine-internally (no board W command). Full standby requires explicit W0000=0x00 from the board.

## App/Cloud-Only Settings — No Gicar Writes Observed

These settings generated zero W commands on the B-channel:

- Auto-standby time (09_standby_time_change): 20→30→20 min produced no Gicar writes. Standby timing is stored on the original board's NVS or managed in the LM cloud.

- Schedule (10_schedule_change): 05:00→06:00 produced no Gicar writes. Schedule is cloud/app-only. The W00E1=0x81 seen in that capture is a steam-sync write (board re-asserting steam boiler state on app reconnect), not a schedule command.

Implication: Auto-standby time and scheduled on/off cannot be replicated via Gicar alone. Implement in firmware independently (inactivity timer, RTC-based schedule).

## Payload Temperature Tracking

payload[28..29] = coffee boiler temperature — confirmed live sensor reading, NOT setpoint echo.

Confirmed from idle captures (±1°C oscillation around setpoint as heater cycles):

0x03AC = 940 → 94.0°C   (at 94°C setpoint)

0x03A2 = 930 → 93.0°C   (same setpoint, heater off cycle)

0x03B6 = 950 → 95.0°C   (at 95°C setpoint)

Confirmed from 14_temp_shot (2026-06-22 — empty basket shot, ~15s):

0x0366 = 870 → 87.0°C   brew=1, p[27]=0x7B

8°C drop from 95°C baseline to 87.0°C during empty basket shot. Cold water flowing through an unloaded group head cooled the boiler 8°C — a setpoint register would not move. This definitively confirms payload[28..29] is the live thermocouple/PT100 reading.

During the 15s brew capture: 162 autonomous Z-frames filled 8,910 of 8,960 M-channel bytes (~99% bandwidth). Only 1 R40000023 poll response got through. This explains why temperature tracking during a shot requires either Z-frame decoding or polling at a faster rate.

## Backflush Sequence (07_backflush_cycle)

5 W commands captured in first 10s (30s window; full cycle is ~130s):

t=  5246ms  W 00E1 0001 82

t=  6246ms  W 00E1 0001 82

t=  7258ms  W 00E1 0001 82

t=  9258ms  W 00E1 0001 82   ← 2s pause then resume

t= 10259ms  W 00E1 0001 82

6 commands confirmed in first 30s of the capture at ~1s intervals. Full cycle is ~130s so the actual sequence runs much longer. Our machine_trigger_clean() sends a single 0x82 — to fully replicate backflush, need to call it repeatedly at ~1s intervals for ~130s under app control.

## Capture File Summary

| File | Bytes (B/M) | R-frames | Key W commands |
| --- | --- | --- | --- |
| 01_idle.txt | 539 / 3862 | 37 | none |
| 02_brew_active.txt | 22 / 16756 | 0 (autonomous) | none |
| 03_steam_active.txt | 540 / 3862 | 37 | none |
| 04_temp_change.txt | 591 / 4038 | 35 | W0007=0x03B6 (95°C) |
| 05_preinfusion_change.txt | 563 / 3894 | 36 | W00E2=0x14 |
| 06_preinfusion_off.txt | 563 / 3923 | 35 | W000B=0x00 |
| 07_backflush_cycle.txt | — / 55363 | 62 | W00E1=0x82 ×6 (in 30s window) |
| 08_steam_boiler_off_on.txt | — / 4050 | 39 | W00E1=0x01 |
| 09_standby_time_change.txt | 539 / 3835 | 36 | none |
| 10_schedule_change.txt | 563 / 3956 | 38 | W00E1=0x81 (sync) |
| 13_power_on.txt | 803 / 5808 | 37 | boot sequence (see above) |
| 14_temp_shot.txt | — / 8960 | 1 | none (brew only) |
| 15_preinfusion_shot.txt | — / 10622 | 2 (pre-brew) | none (brew only) |

## Known Gaps

- Pre-infusion ON write: 0x000B=0x01 not captured (only the OFF=0x00 write was observed).

- Pre-infusion duration unit: 0x14=20 — seconds or tenths unknown without cross-reference.

- Brew autonomous frame format: Z-frame structure now decoded (see above). Temperature at (pair[46,47]×256+pair[48,49])/160°C confirmed. Shot-active flag at pair[34,35]. Pre-infusion phase detectable via temperature plateau. Pair[40,41] = rolling frame counter.

- 0x0300/0x0310 blocks: contents unknown (read-only by board, never written).

- Standby/schedule via Gicar: confirmed not possible without test-mode/unknown registers.